Date: 19 Sep 2010

Part No.: PT80453

## CU/MICA&LSZH/OS/FRPVC/SWA/VERMIN-PROOF FRPVC

## FIRE RESISTANT TELEPHONE CABLE

Application:

This cable is used for communication and telephone systems as main and distribution cabling. Armored type is offered for direct burial & harsh environment installations due to it additional mechanical protection.

Construction:

Copper Conductor

Mica & LSZH Insulation

Al Overall Screen

FRPVC Inner Jacket

Steel Wire Armor

Vermin-proof FRPVC Outer Jacket

Conductor: 			   Plain Annealed Solid Copper to IEC 228 Class 1

Insulation:  			   Mica and LSZH insulation

Shape: 				   Circular

Overall Screen:        Overall Polyester/ Aluminium foil screened plus copper drain wire

Bedding:              FRPVC

Armour:              Galvanized Steel Wire Armour

Outer Sheath:		   Vermin-proof FRPVC

Standard:             IEC60332 & IEC60331

Marking:  The cable is marked with white or other colors as follows:

- Caledonian Cables Co., Ltd

- Type and size of cable

- Manufacturing date

- Meter Marking

The meter mark is printed every meter with an accuracy of 0.5%.

Other printing are available according to customer request.

Table 1. Technical Data

| Number of Pairs | Diameter of Conductor (mm) | Nominal Dia under Armour (mm) | Armour Wire Dia (mm) | Thickness of Outer Jacket (mm) | Diameter of Cable (mm) |
| --- | --- | --- | --- | --- | --- |
| 4 | 0.9 | 19.0 | 1.5 | 1.8 | 25.6 |
| 25 | 0.9 | 42.2 | 2.0 | 2.5 | 51.2 |

The data included in the present catalogue are merely indicative; Caledonian Cables Limited reserves to itself the right to change them as its own discretion in any time.