Date: 17 May 2010

## Part No.: PT80309

## 0.5MM CU/MICA/LSZH/LSZH/STA/LSZH

## CATEGORY 3 FIRE RESISTANT TELEPHONE CABLE

Application:

This cable is used for medium frequency data or telephone systems, suitable for indoor and outdoor installations.

Copper Conductor

Mica and LSZH Insulation

Steel Tape Armour

LSZH Inner Jacket

LSZH Outer Jacket

Construction:

Conductor: 			   Plain Annealed Solid Copper to IEC 228 Class 1

Insulation:  			   Mica tape and LSZH insulation

Shape: 				   Circular

Inner Sheath:          LSZH

Armour:              Steel tape armour

Outer Sheath:		   LSZH

Sheath color:          Black

Voltage Rating:        300/500V

Temperature Rating:   0 ℃ to 40 ℃

Bending Radius:       6 x overall diameter

Marking:  The cable is marked with white or other colors as follows:

- Caledonian Cables Co., Ltd

- Type and size of cable

- Manufacturing date

- Meter Marking

The meter mark is printed every meter with an accuracy of 0.5%.

Other printing is available according to customer request.

Table 1. Technical Data

| Number of Pair | Diameter of Conductor (mm) | Diameter of Cable (mm) |
| --- | --- | --- |
| 50 | 0.5 | 30.0 |

Electrical Properties:

Frequency: 1—16MHz

Characteristic Impedance: 100±15Ω
Nominal Velocity of Propagation： 65%
Maximum DC. Resistance： 9.38Ω/100m
Maximum DC Resistance Unbalance： 5%
Maximum Propagation Delay Skew： 30ns / 100m
Maximum Propagation Delay： 536 ns / 100m @ 100 MHz
Minimum Bending Radius: 10 × cable outer diameter
Voltage Rating： 60V rms
Maximum Pulling Load: 80 N
Working Temperature： (-20) ℃ — (+ 60) ℃

Storage Temperature ： (-5) ℃ — (+50) ℃ 
Flame Retardancy： IEC 332-3A, IEC60331

Table 2. Transmission Parameters

|  | NEXT (db/100m ） Minimum Value/Typical Value/Standard Value ≥ | ATTENUATION (db/100m ） ≤ | SRL (db) Minimum Value/Typical Value/Standard Value ≥ |
| --- | --- | --- | --- |
| 1 | 43.0/48.0/41.0 | 2.6 | 13.0/16.0/12.0 |
| 4 | 34.0/38.0/32.0 | 5.6 | 13.0/16.0/12.0 |
| 8 | 29.0/33.0/26.0 | 8.5 | 13.0/16.0/12.0 |
| 10 | 28.0/33.0/26.0 | 9.8 | 13.0/16.0/12.0 |
| 16 | 25.0/30.0/23.0 | 13.1 | 11.0/14.0/10.0 |

The data included in the present catalogue are merely indicative; Caledonian Cables Limited reserves to itself the right to change them as its own discretion in any time.